NanoLumens

R&D Dark Room Sign

Project Report & Troubleshooting Guide

Author: Yiwei B. (nano_yiweib)

Date: July 2026

Version 1.0

# 1. Project Overview

This report documents the design, implementation, and troubleshooting of the NanoLumens R&D Dark Room Sign system — a dual-display room status sign installed outside the R&D dark room. The system allows test engineers inside the lab to communicate the room's current status to colleagues outside without interrupting ongoing testing.

## 1.1 Objectives

- Provide a clear, visible room status sign outside the R&D dark room.

- Allow test engineers to change the status from inside using a Bluetooth numpad without leaving the room.

- Display a real-time clock and multiple configurable status messages.

- Run 24/7 with automatic startup on power-on, requiring no manual intervention.

- Provide an internal mirror display (e-ink hat) so the engineer inside can confirm what the sign shows outside.

## 1.2 Hardware Components

| Component | Details |
| --- | --- |
| Controller | Raspberry Pi Zero WH (single-core 1GHz ARM11, 512MB RAM) |
| Inside Display | Waveshare 2.13" e-Paper HAT V4 (250×122 px, B&W, SPI) |
| Outside Display | 160×40 LED sign panel via HDMI + media controller |
| Input Device | Havit SMART26 Bluetooth Numeric Keypad (MAC: DC:2C:26:A2:E3:AD) |
| Storage | Micro SD card with Raspberry Pi OS (32-bit, Debian Trixie) |
| Power | 5V 3A USB charger via micro USB PWR IN port |

# 2. System Architecture

The system consists of two independent Python scripts running as systemd services, both listening to the same Bluetooth numpad for input:

- sign_eink.py — Drives the Waveshare e-ink HAT via SPI (inside mirror). Updates only when mode changes (no refresh penalty).

- sign_hdmi.py — Drives the 160×40 LED panel via HDMI using pygame with SDL KMS/DRM driver. Updates on mode change, clock tick, or flash toggle.

- Both scripts share the same key mapping and run independently — pressing a numpad key updates both displays simultaneously.

## 2.1 Software Stack

| Library / Tool | Purpose |
| --- | --- |
| Python 3.13 | Main scripting language for both display scripts |
| pygame 2.6.1 | HDMI panel rendering via SDL KMS/DRM framebuffer |
| PIL / Pillow | E-ink image rendering and text layout |
| waveshare_epd | Waveshare e-Paper HAT V4 driver (epd2in13_V4) |
| evdev | Bluetooth numpad input event reading |
| systemd | Service management and auto-start on boot |
| BlueZ | Bluetooth stack — handles numpad pairing and reconnection |

# 3. Numpad Key Mapping

The Havit SMART26 Bluetooth numpad controls both displays simultaneously. Key mapping is as follows:

| Key | Message | HDMI Color | Notes |
| --- | --- | --- | --- |
| 1 | Clock (HH:MM:SS) | Cyan | Live seconds, updates every second |
| 2 | TEST IN PROGRESS / DO NOT ENTER | White / Red flashing | Thick border, flashing border |
| 3 | OCCUPIED / OTHER TESTING | White / Red flashing | Thick border, flashing border |
| 4 | ROOM NOT AVAILABLE | White / Amber | No divider line |
| 5 | OPEN / COME ON IN | White / Green | Thin border |
| 6 | ON BREAK / BACK SOON | White / White | Thin border |
| 7 | COME BACK IN / 5 MIN | White / White | Thin border |
| 8 | COME BACK IN / 10 MIN | White / White | Thin border |
| 9 | COME BACK IN / 20 MIN | White / White | Thin border |
| 0 | R&D DARK ROOM (idle) | White | Default/idle state |
| Enter | Blank / Off | Black | Clears both displays |
| * | Reserved (no-op) | — | Future: brightness control |
| / | Reserved (no-op) | — | Future: toggle |

# 4. Setup Guide

## 4.1 Flash the SD Card

- Use Raspberry Pi Imager → Raspberry Pi OS (32-bit, Debian Trixie with Desktop).

- In the OS Customisation settings: set hostname to darkroom-sign, enable SSH, set Wi-Fi credentials, set username/password.

- Flash the card and insert into the Pi Zero WH.

## 4.2 Configure /boot/config.txt

Before first boot, edit config.txt on the bootfs partition and add the following at the bottom (inside the [all] section):

hdmi_force_hotplug=1
dtoverlay=vc4-kms-v3d
hdmi_cvt=160 40 60 1 0 0 0

Also comment out dtoverlay=vc4-kms-v3d and disable_fw_kms_setup=1 from the middle of the file to avoid duplication.

## 4.3 First Boot & Dependencies

- SSH in: ssh nano_yiweib@darkroom-sign.local

- Enable SPI: sudo raspi-config → Interface Options → SPI → Yes

- Switch to console autologin: sudo raspi-config → System Options → Boot → Console Autologin

- Install dependencies: sudo pip3 install unicornhatmini evdev pygame --break-system-packages

- Install PIL: sudo apt install python3-pil python3-numpy -y

- Clone Waveshare library: git clone https://github.com/waveshare/e-Paper.git /home/nano_yiweib/e-Paper

- Copy library to project: cp -r /home/nano_yiweib/e-Paper/RaspberryPi_JetsonNano/python/lib/waveshare_epd /home/nano_yiweib/led-sign/

## 4.4 Bluetooth Numpad Pairing

- Unblock Bluetooth: sudo rfkill unblock bluetooth

- Open bluetoothctl: sudo bluetoothctl

- Run: power on → agent on → scan on

- Put the Havit SMART26 into pairing mode and wait for MAC DC:2C:26:A2:E3:AD to appear.

- Run: pair DC:2C:26:A2:E3:AD → trust DC:2C:26:A2:E3:AD → connect DC:2C:26:A2:E3:AD → quit

- BlueZ handles reconnection natively when the numpad wakes from sleep — no custom watchdog needed.

## 4.5 Deploy Scripts

- Create project directory: mkdir -p /home/nano_yiweib/led-sign

- Copy scripts: scp sign_eink.py sign_hdmi.py nano_yiweib@darkroom-sign.local:/home/nano_yiweib/led-sign/

- Test e-ink manually: python3 sign_eink.py (stop led-sign.service first if running)

- Test HDMI manually: SDL_VIDEODRIVER=kmsdrm python3 sign_hdmi.py

## 4.6 Install systemd Services

Two services are required — one for each display script:

sudo cp led-sign.service /etc/systemd/system/
sudo cp led-sign-hdmi.service /etc/systemd/system/
sudo systemctl daemon-reload
sudo systemctl enable led-sign led-sign-hdmi
sudo systemctl start led-sign led-sign-hdmi

Verify both services are running: sudo systemctl status led-sign led-sign-hdmi

Important: verify symlinks exist in multi-user.target.wants (not graphical.target.wants):

ls -la /etc/systemd/system/multi-user.target.wants/ | grep led

# 5. Troubleshooting Guide

## 5.1 E-Ink Display Issues

Problem: lgpio.error: GPIO busy

Cause: Another process (usually led-sign.service) is already holding the SPI/GPIO pins.

Fix:

- sudo systemctl stop led-sign

- Then run the script manually.

Problem: E-ink display not updating

Cause: SPI not enabled, or driver version mismatch.

Fix:

- Verify SPI is enabled: sudo raspi-config → Interface Options → SPI

- Confirm correct driver: test with epd2in13_V4 — this is the confirmed working driver for this display

- Check waveshare_epd folder exists in /home/nano_yiweib/led-sign/

## 5.2 HDMI Panel Issues

Problem: pygame.error: video system not initialized

Cause: SDL cannot find a display, usually because the service is using X11 driver but no desktop session is running.

Fix:

- Confirm SDL_VIDEODRIVER=kmsdrm is set in led-sign-hdmi.service (not x11)

- Confirm SDL_FBDEV=/dev/fb0 is set in the service file

- Confirm the Pi is booting to console (not desktop): sudo raspi-config → System Options → Boot → Console Autologin

- Verify user is in video and render groups: groups nano_yiweib

Problem: HDMI service shows inactive (dead) after boot

Cause: Service symlink is in graphical.target.wants instead of multi-user.target.wants.

Fix:

- sudo systemctl disable led-sign-hdmi

- sudo systemctl enable led-sign-hdmi

- sudo systemctl daemon-reload

- Verify: ls -la /etc/systemd/system/multi-user.target.wants/ | grep led

Problem: HDMI shows desktop or wrong resolution

Cause: A regular monitor is connected instead of the 160×40 panel, or config.txt edits were not saved correctly.

Fix:

- Connect only the 160×40 LED panel — KMS will auto-detect its native resolution

- Verify config.txt has hdmi_force_hotplug=1 and hdmi_cvt=160 40 60 1 0 0 0 in the [all] section

## 5.3 Bluetooth Numpad Issues

Problem: Bluetooth fails to power on — "Failed to set mode: Failed (0x03)"

Cause: Bluetooth radio is soft-blocked by rfkill.

Fix:

- sudo rfkill unblock bluetooth

- sudo hciconfig hci0 up

- This is handled automatically by led-sign.service via ExecStartPre=/usr/sbin/rfkill unblock bluetooth

Problem: Numpad not detected / "Still looking for numpad..."

Cause: Numpad is asleep (normal), not paired, or Bluetooth not initialized yet.

Fix:

- Press any key on the numpad to wake it — BlueZ will auto-reconnect within a few seconds

- Verify pairing: bluetoothctl info DC:2C:26:A2:E3:AD — should show Paired: yes, Trusted: yes

- If not paired, re-pair via bluetoothctl (see Section 4.4)

Problem: OSError: No such device — numpad thread crashes

Cause: Numpad went to sleep and the evdev device file was removed while the script was reading it.

Fix:

- This is handled in both scripts via OSError recovery loop — the input thread will automatically restart and look for the numpad again

- If both scripts are on the latest version (with the recovery loop), this should self-heal without intervention

## 5.4 Boot & System Issues

Problem: Pi freezes on boot (solid green LED)

Cause: System overloaded during boot — services starting too early competing with OS initialization.

Fix:

- Unplug and replug power — wait the full 3-4 minutes before judging

- If recurring: increase ExecStartPre sleep values in both service files (led-sign: 25s, led-sign-hdmi: 25s)

Problem: Cannot SSH in — "Connection refused" or "No such host"

Cause: Pi still booting, on a different network, or mDNS not resolving.

Fix:

- Wait 3-4 minutes after power-on before attempting SSH

- Try by IP address: ssh nano_yiweib@10.26.11.122 (IP may vary — check router)

- Verify Pi is on the same Wi-Fi network (Pi Zero WH is 2.4GHz only)

Problem: Wi-Fi drops after running nmcli command

Cause: nmcli briefly drops the connection when applying new credentials.

Fix:

- Wait 60 seconds and try SSH again — it usually reconnects automatically

- If still disconnected, remove SD card, create wpa_supplicant.conf on bootfs partition with correct SSID/password, reinsert and reboot

# 6. Service Management Reference

| Command | Purpose |
| --- | --- |
| sudo systemctl status led-sign | Check e-ink service status |
| sudo systemctl status led-sign-hdmi | Check HDMI service status |
| sudo systemctl restart led-sign | Restart e-ink service |
| sudo systemctl restart led-sign-hdmi | Restart HDMI service |
| sudo systemctl stop led-sign | Stop e-ink service (e.g. before manual testing) |
| sudo journalctl -u led-sign -f | Watch e-ink service logs live |
| sudo journalctl -u led-sign-hdmi -f | Watch HDMI service logs live |
| sudo shutdown -h now | Clean shutdown (always use instead of unplugging) |
| sudo reboot | Reboot the Pi |

# 7. Pending & Future Work

- 3D print enclosure for Pi Zero WH + Waveshare 2.13" e-ink HAT stack — search "Raspberry Pi Zero Waveshare 2.13 e-ink case wall mount" on Thingiverse or Printables.

- Design wall mount bracket for the 160×40 LED panel outside the dark room door.

- Implement Flask or FastAPI web interface for editing sign messages without SSH — accessible from any browser on the lab network.

- Wire up reserved numpad keys (* and /) — brightness cycle and blink toggle.

- Investigate scrolling text animation for the HDMI panel on the testing mode (currently static).

# 8. File Inventory

| File | Description |
| --- | --- |
| /home/nano_yiweib/led-sign/sign_eink.py | E-ink HAT display script (inside mirror) |
| /home/nano_yiweib/led-sign/sign_hdmi.py | HDMI panel display script (outside sign) |
| /home/nano_yiweib/led-sign/waveshare_epd/ | Waveshare e-Paper driver library |
| /etc/systemd/system/led-sign.service | systemd service for e-ink script |
| /etc/systemd/system/led-sign-hdmi.service | systemd service for HDMI script |
| /boot/config.txt | Pi boot config (HDMI resolution, KMS driver) |

NanoLumens R&D — Internal Documentation — July 2026